multiple clock low power system RTL to GDS

System specs

Test bench snippets  for some of the test cases

Reserved registers data is saved correctly as mentioned in the specs

Clock relations

Gated clk is on when the alu is doing its operation but it is turned on before the alu calulates the result to be stable and ready

Data is stored in address 5 in reg file as expected

Output of data synchronizer between the uart rx and system control :

Command code = 0xaa in this state the RF_WR_CMD_FLAG is set to high

write address  = 0x05 in this state the address is registered until the write data arrives in the next data

write data  = 0x19 (8’d25) in this state the sys_ctrl  set the w_en of the reg file to high then stores the data in address 5 in the reg_file

Readed data from the register file is as expected (8’d129)

Output of data synchronizer between the uart rx and system control :

Command code = 0xbb in this state the RF_RD_CMD_FLAG is set to high

read address  = 0x02

- The readed data is then stored in the fifo until the uart tx read it and transmit it as follows :start bit , data bits , even parity bit , stop bit  and the busy signal is high as long as the frame is being sent

- the data valid signal of the tx is set to high when there is any data stored in the fifo

- The output of the alu is 16 bits and the fifo data width is 8 bits so the fifo stores any data comes from the alu in two 8-bits locations  then  the the uart tx transmits  the 8-LSB first in a single frame then transmits the 8-MSB in another single frame (each frame of them has start , data , parity and stop bits)

- Busy signal is high from the start state to the stop state and then goes low in idle state

another single frame (each frame of them has start , data , parity and stop bits)

Output of alu is as expected

(11 +15 = 25)

Output of data synchronizer between the uart rx and system control :

Command code = 0xcc

Operand a value   = 0x0b (stored in address 0 in reg file by default)

Operand b value   = 0x0f (stored in address 1 in reg file by default)

Alu_fun value   = 0x00 (do addition)

- The output of the alu is 16 bits and the fifo data width is 8 bits so the fifo stores any data comes from the alu in two 8-bits locations  then  the the uart tx transmits  the 8-LSB first in a single frame then transmits the 8-MSB in another single frame (each frame of them has start , data , parity and stop bits)

- Busy signal is high from the start state to the stop state and then goes low in idle state

Output of alu is as expected

(127*253 = 32131)

Output of data synchronizer between the uart rx and system control :

Command code = 0xdd

Operand a value   = this command reads the value stored in address 0 in the reg file which is 8’d127

Operand b value   = this command reads the value stored in address 1 in the reg file which is 8’d253

Alu_fun value   = 0x02 (do multiply)

synthesis

Worst setup path

Worst hold path

Master and generated clocks

Total area

Total power

Synthesis gate level netlist snippets

Post synthesis formality

DFT

Calculate number of chains

Worst setup path

Worst hold path

Master and generated clocks

Total area

Total power

DFT gate level netlist snippets

Scan chain distribution

Coverage percentage

Post dft formality

PNR

Floor planning

Power planning

Placement

Add filler cells

Routing (post route)

Check connectivity

Check geometry

Check if there are any gaps after adding filler cells

Check drc violations

Check process antenna

Setup and hold timing analysis (post route)

- Link for the rtl code of the system and all backend scripts and files :  github link

- Link for the rtl code and functional verification of  each block :github link